Use Case Specifications: LEED Certification & Peer Benchmarking

New tool category for EnergyPlus-MCP: Sustainability & Certification Scoring

# Use Case 1 — LEED Certification (EA Credit: Optimize Energy Performance)

Determines how many points a building earns toward LEED certification based on measured energy performance improvement over the ASHRAE 90.1 code baseline, and what to change to earn more.

## Building manager questions this use case answers

- How close are we to LEED Gold / Silver / Platinum right now?

- Which credits are we missing, and how many points do we still need?

- Are we meeting the minimum energy performance prerequisite (EAp2)?

- If we upgrade the HVAC system or add window film, will it push us to the next certification tier?

- What is the cheapest way to gain 5 more points before our LEED submission deadline?

## Input

- Loaded IDF model + weather file

- Target certification tier (Certified / Silver / Gold / Platinum)

- Metric to use: cost or source energy (per LEED v4 / v4.1 rules)

## Output

- Percentage performance improvement over the ASHRAE 90.1 Appendix G baseline

- Points earned for this credit, plus running total score across categories

- Predicted certification tier

- List of remaining gap credits, ranked by effort

## Episode — tool sequence

| Step | Tool | Existing / New | What it does / formula |
| --- | --- | --- | --- |
| 1 | load_idf_model, get_model_summary | Existing | Loads the model; pulls site location and climate zone |
| 2 | generate_ashrae_baseline_model | New | Builds an ASHRAE 90.1 Appendix G baseline version of the model: standard envelope U-values, 40% max window-to-wall ratio, code-minimum HVAC system type by building size/type, orientation rotated 4 ways and averaged. Required before Step 4 can calculate anything. |
| 3 | add_output_meters, run_energyplus_simulation (x2) | Existing | Runs the baseline model and the proposed (actual) model, capturing annual energy cost and/or source energy and GHG emissions |
| 4 | map_to_certification_credits | New | % Improvement = (Baseline Performance - Proposed Performance) / Baseline Performance x 100. Looks up the % against the official LEED point table (version/year aware, since thresholds changed in the March 2024 update). Baseline and proposed models must use identical GHG emission factors per fuel source; points are the sum of Table 1 (cost/source energy) and Table 2 (GHG emissions). |
| 5 | identify_quick_wins | New | Sorts not-yet-achieved credits by points-per-effort (points gained divided by estimated modeling/construction effort) |
| 6 | simulate_certification_impact | New | Re-runs the proposed model with a hypothetical change (e.g., add window film) and recalculates the % improvement and point delta from Step 4 |

## Notes

generate_ashrae_baseline_model is listed here as its own tool (not an internal helper) because map_to_certification_credits cannot calculate anything without a valid baseline model, and building the baseline correctly (Appendix G envelope rules, 4-way orientation rotation and averaging, code-minimum HVAC selection) is substantial enough work to warrant its own tool with its own tests.

Only EA-category credits are computed directly from simulation. Other LEED categories (Location & Transportation, Sustainable Sites, most of Materials & Resources) depend on site/GIS data or procurement documentation outside EnergyPlus's scope, and are not covered by this use case.

# Use Case 2 — Peer / EUI Benchmarking

Compares a building's actual energy use intensity (EUI) against a peer set of similar buildings, and suggests what EUI target and upgrades would move it into a better percentile.

## Building manager questions this use case answers

- How does our building's energy use compare to similar buildings in this climate?

- Are we in the top half or bottom half of our building type and size category?

- What EUI would we need to hit to be considered top-performing, or to reach an ENERGY-STAR-equivalent benchmark?

- Is our building underperforming relative to peers of the same size and use, or is our EUI actually reasonable for our climate zone?

- Which single upgrade gives us the biggest jump in our peer ranking?

## Input

- Building EUI (from simulation)

- Building type, gross floor area, climate zone

- A peer building dataset to compare against (CBECS-style, or a regional equivalent - India does not have a public CBECS-equivalent dataset, so this needs a decision: build a small reference dataset from ECBC-compliant buildings, or scope to a specific climate zone/building type where comparable data exists)

- Target percentile (e.g., 75th percentile, the ENERGY STAR certification threshold)

## Output

- Site EUI and Source EUI (kWh/m2/yr)

- Percentile rank against the peer set

- Suggested target EUI to reach the desired percentile

- Ranked list of ECMs (energy conservation measures) to close the gap

## Episode — tool sequence

| Step | Tool | Existing / New | What it does / formula |
| --- | --- | --- | --- |
| 1 | load_idf_model, get_model_summary | Existing | Loads the model; pulls building type and climate zone |
| 2 | add_output_meters, run_energyplus_simulation | Existing | Gets annual energy consumption by fuel type and end-use |
| 3 | benchmark_eui_against_peers | New | Site EUI = Total Annual Site Energy (kWh) / Gross Floor Area (m2). Source EUI = Sum(Site Energy per fuel x source-to-site conversion factor) / GFA. Queries the peer dataset and computes Percentile Rank = (number of peer buildings with higher EUI / total peer buildings) x 100 |
| 4 | suggest_target_score | New | Given a target percentile (e.g., 75th, the ENERGY STAR certification threshold), interpolates against the peer distribution to find the EUI value at that percentile, then reports Required Reduction = Current EUI - Target EUI |
| 5 | identify_quick_wins | Shared with Use Case 1 | Ranks ECMs (energy conservation measures) by expected EUI reduction per unit of effort/cost |
| 6 | simulate_certification_impact | Shared with Use Case 1 | Re-simulates with a proposed ECM and recomputes EUI and percentile shift |

## Notes

The percentile-rank formula above works directly off a raw peer dataset. ENERGY STAR's actual 1-100 score instead uses a regression model (predicted energy use as a function of climate, occupancy hours, and building characteristics, then a percentile of the residual) rather than a raw EUI comparison. Worth deciding early whether to build the simpler raw-percentile version (easier to build, still defensible) or the regression version (matches industry standard, but needs a larger dataset).

# Combined Tool Inventory

Six new tools are needed in total across both use cases; four of the five originally scoped tools map directly, plus the baseline-model generator identified as a hard dependency for Use Case 1.

| Tool | Status | Used in |
| --- | --- | --- |
| load_idf_model, get_model_summary, add_output_meters, run_energyplus_simulation | Existing (EnergyPlus-MCP, 35-tool set) | Both use cases |
| generate_ashrae_baseline_model | New (6th tool - required dependency) | Use Case 1 (LEED) |
| map_to_certification_credits | New | Use Case 1 (LEED) |
| benchmark_eui_against_peers | New | Use Case 2 (Benchmarking) |
| suggest_target_score | New | Use Case 2 (Benchmarking), reusable for LEED tier targets |
| identify_quick_wins | New | Both use cases |
| simulate_certification_impact | New | Both use cases |